IWRAP models of two areas in the Baltic

Summary

In this note two fairway areas (Great Belt of Denmark and the Gulf of Finland) in the Baltic Sea are modeled using the Integrated Waterway Risk Assessment Programme (IWRAP) tool developed and used i.a. by the International Association of Aids to Navigation and Lighthouse Authorities (IALA). The purpose is to estimate the number of collisions and groundings today and in the future (2030). The number of ships in 2030 is found using the values for increase in traded volumes published in the Baltic Transport Outlook 2030. This does not account for the fact that some ships will probably increase in size and that the ships of 2030 will most likely be safer than the ships of 2010. The estimated collision and grounding frequencies should therefore be considered the worst case if no improvements happen. It is found that the absolute number of collisions increases by 80 % and the absolute number of groundings increases by 40 %. However when normalized by the number of ships, the increase in collisions is 40% and the increase in groundings is 10 %. Grounding analysis has only been carried out for the Great Belt area. Only vessels equipped with Automatic Identification System (AIS) have been included.

Method

The models have been created using the IALA risk tool IWRAP. The method uses the actual traffic volume and traffic pattern to calculate the number of geometrical collisions (The number of collisions that would occur if the navigator sails blindfold).  This number is then multiplied by a causation factor. The causation factor expresses the probability that the navigator does not do as he or she is supposed to do. It is in the order of 10-4. So once or twice in 10,000 situations things go wrong. The outcome of the model is the number of collisions/groundings per year. The methods have been developed over the years starting by Fuji in 1974, latter by Petersen and Friis-Hansen.

To model the situation in 2030 it is assumed that the routes and sailing pattern is unchanged. The number of ships in 2030 is found using the values in Baltic Transport Outlook 2030. It is here assumed that the increase in traded maritime volume correspond to an increase in the number of ships. It is, however, very likely that many of the ships will increase in size instead.

Factor to multiply the number of ships in 2011 to get the number of ships in 2030

Gulf of Finland

Density plot and corresponding legs with lateral distributions 2011

Traffic 2011 on leg 2 and leg 28. The outer left legs in the figure above

Gulf of Finland results

In order for the IWRAP model to reproduce the actual historical collision values, the causation factors for the passenger ships crossing the Gulf of Finland have been increased by a factor  10 compared to the default causation values. Whether this is correct needs to be discussed with experts in the region.

|  | Per year |  | Increase |  | Return period |  |  | Per 10000 ships |  | Increase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2011 | 2030 |  |  | 2011 | 2030 |  | 2011 | 2030 |  |
| Collisions (default factors) | 0.04 | 0.10 | 2.5 |  | 28 | 10 |  | 0.01 | 0.02 | 2.0 |
| Collisions (adjusted factors) | 0.13 | 0.24 | 1.8 |  | 8 | 4 |  | 0.04 | 0.05 | 1.3 |
|  |  |  |  |  |  |  |  | (34000) | (48000) | 1.4 |

It is seen that the absolute number of collisions increases by 80 %. However, when normalized by the number of ships (last 3 columns) the increase is “only” 30 %.

Great Belt

|  | 2011 values for leg going under the bridge |
| --- | --- |

Great Belt results

The calculated number of groundings fits well with the historic values as does the calculated number of collisions. Given the perhaps naive assumption that the ships of 2030 are the same as those of 2011, we will experience a 40% increase in the number of powered groundings and an 80% increase in the number of collisions.

|  | Per year |  | Increase |  | Return period |  |  | Per 10,000 ships |  | Increase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2011 | 2030 |  |  | 2011 | 2030 |  | 2011 | 2030 |  |
| Powered groundings | 1.14 | 1.54 | 1.4 |  | 0.88 | 0.65 |  | 0.38 | 0.41 | 1.1 |
| Collisions | 0.26 | 0.48 | 1.8 |  | 3.8 | 2.1 |  | 0.09 | 0.13 | 1.4 |
|  |  |  |  |  |  |  |  | (30,000) | (38,000) | 1.3 |

As for the Gulf of Finland the number of collisions in the Great Belt increases by 80 % by year 2030. But when normalized by the number of ships the increase is “only” 40 %. 
The number of groundings increases by 40 %, but when normalized by the number of ships “only” by 10 %.

http://www.baltictransportoutlook.eu/

http://www.ialathree.org/iwrap Here a detailed description on the method can be found.